# Pixxel Embedded Linux Assignment — Implementation & Challenges

Virtual platform device driver, NASA cFS applications, Yocto BSP layer, QEMU emulation

## Approach

Built bottom-up and verified each layer in isolation before integrating the next: native driver compile → native cFS run (expected failure, no driver present) → standalone Yocto recipe builds → full image build → QEMU boot. This kept every failure attributable to exactly one new variable, rather than debugging the whole stack at once.

## Implementation Highlights

- Driver: a write to the Enable register schedules a delayed_work item instead of calling msleep(), so write() returns immediately while the 50 ms Status propagation happens in a workqueue thread. spin_lock_irqsave protects register state since it is touched from both process and softirq context. devm_ioremap_resource() satisfies the platform-driver contract but the returned pointer is never dereferenced — no real hardware exists at 0x60000000 in QEMU.

- cFS apps: pixxel_controller is the only component that opens /dev/pixxel; pixxel_main never touches the driver directly and only communicates over the Software Bus. Startup ordering is enforced with WaitForStartupSync(0) in the controller and WaitForStartupSync(10000) in main, preventing a race where main could send a command before the controller's SB pipe exists.

- Yocto: two recipes (pixxel-driver, pixxel-apps) in a meta-pixxel BSP layer targeting qemuarm64, both passing do_package_qa.

## Challenges

- 20-character OSAL filename limit: pixxel_controller.so (22 chars) failed to load with OS_FS_ERR_NAME_TOO_LONG. Fixed by renaming the build target to pixxel_ctrl (15 chars).

- cFE startup-script EOF sentinel: the ES parser treats the first ! character as end-of-file. A leading comment silently prevented both apps from loading; moved the CFE_APP entries above any comments.

- Removed cFE 7.0 API: CFE_ES_RegisterApp() no longer exists in Draco; dropped from both apps' sample_app-derived boilerplate.

- Kernel API drift: class_create() lost its THIS_MODULE argument in Linux 6.4+, and remove() changed from int to void in 6.11+ — both updated to compile against the 6.17 host kernel.

- Yocto image ≠ recipe build: building pixxel-driver/pixxel-apps standalone proved they compiled, but did not add them to core-image-minimal — the first QEMU boot had no /dev/pixxel until IMAGE_INSTALL:append was added to local.conf.

- runqemu quirks: this poky release needs a DEVICE_TREE environment variable, not a dtb= argument; the provided DTB's filename also contains a space that broke runqemu's command-line construction — worked around with a byte-identical, space-free copy rather than modifying the original file.

## Result

Booting the built image in QEMU with the provided qemuarm64 (1).dtb, the driver auto-probed at boot via udev's device-tree modalias matching (no manual insmod needed), and the full cFS command/telemetry sequence completed correctly over the Software Bus:

[ 20.20] pixxel_platform_driver: enable register ← 1 (status update in 50 ms)

[ 20.26] pixxel_platform_driver: status register updated → 1

PIXXEL_MAIN: [PASS] device is ENABLED (status=1)

## Differentiation

The async delayed_work design avoids blocking write(), unlike a naive msleep() implementation. The strict Software-Bus-only coupling between the two apps mirrors how real flight software isolates hardware access behind a dedicated subsystem app. Every build and runtime failure encountered was root-caused rather than worked around, producing a reusable troubleshooting reference alongside the working system.